Title? Intro?

| Basic Physical Layer Parameters |  |
| --- | --- |
| Sampling Rate | 48,000 Hz |
| FFT Size N | 8192 |
| Cyclic Prefix - CP | 2048 |
| Band, Pilots & Modulation |  |
| Frequency Band | 2-15kHz |
| Active Subcarriers | k= 400(inclusive)~2392(exclusive) (all subcarriers indexed from 0 to 4095. The content of the remaining carriers is unspecified. ) |
| Training Sequence | 8 Leading + 8 Trailing OFDM Symbols （Generate QPSK data for 16 training symbols using a fixed random seed. Independently draw integers from 0 to 3 and map them as follows: 0 -> 1+j, 1 -> -1+j, 2 -> 1-j, 3 -> -1-j.) |
| Modulation Scheme | QPSK(Anticlockwise, Gray mapping, starting from 00->1+j) |
| Bit & Byte Order | Big-endian |
| Sync Correction | See the code for more details. |
| Synchronization | Identical Front and Rear Chirps |
| Chirps Frequency Band | 0.1k~20kHz |
| Chirp Duration | 3s |
| Chirp type | Linear |
| PHY Header Spec | Located at the beginning of the first payload OFDM symbol; the remaining byte positions are zero-padded (for example zeros，unspecified sequence works) |
| Magic Word | ASCII "PH", 2 bytes |
| Protocol Version | 1 byte |
| Source File Size | Unsigned integer, 4 bytes, byte unit, big-endian |
| Payload Symbol Count | Unsigned integer, 4 bytes, big-endian |
| File Name | Variable-length UTF-8 string, including the file extension |
| File Name Terminator | 0x00, 1 byte |
| Header CRC32 | 4 bytes, big-endian; calculated from the Magic Word through the last file-name byte |
| Padding | the remaining byte positions are zero-padded (for example zeros，unspecified sequence works) |
| Header Combining | Single transmission of the header, no repetition |
| LDPC Format | One LDPC codeword is mapped to one payload OFDM symbol |
| Standard | 802.16 |
| Expansion Factor | Z = 166 |
| Code Rate | 1/2 |
| Base Matrix Size | 12*24 |
| Information Block Length | K = 12Z = 1992 bits = 294 bytes |
| Parity Length | N - K = 1992 bits = 249 bytes |
| LDPC Codeword Length | N = 24Z = 3984 bits = 498 bytes |
| Input Bit Order | MSB-first |
| Padding | Zero padding is added to the final incomplete information block |
| Post-LDPC Processing / LDPC | Scrambling is applied after LDPC encoding |
| Scrambler | See the code for more details. |
| Register Length | 15 bits |
| Initial Seed | 0x5A4D |
| Initial State | 101101001001101 |
| Output Bit | Bit 14, the most significant register bit |
| Feedback | state[14] XOR state[13] |
| Shift Direction | Left shift |
| Scrambling Operation | Bitwise XOR |
| Sequence Reset | The LFSR is reset to 0x5A4D for every function call |

See the code for transmitter implementation details.

ExampleCode - Google 云端硬盘